Lab #4

Objectives

- Analyze systems using Z-transform

Report

- Your report must include your answers in hand-written or computer-aid tools.

- Do not share your report with your friends.

- Finally, you upload your report to BKeL on time.

---------------------------------

Z-TRANSFORM EXERCISES

For each discrete-time signal x(n), find the corresponding Z-Transform and its ROC

Exercise 1.

xn=2δn+2−1δn+1+2δn−3δn−1+4δn−2

Exercise 2.

xn=0.5nun+0.4nun

Exercise 3.

xn=0.5nun+0.9nu−n−1

ADDITIONAL SCILAB EXERCISES

Exercise 4.

Investigate the existing library or toolbox in Scilab to process audio data.

Exercise 5.

Investigate the existing library or toolbox in Scilab for image processing. Then, you can do an example or a demo to show a simple manipulation (e.g., Histogram display, histogram equalization, blur, and watermarking) on a digital image.

Exercise 6.

Let there are two discrete signals xn,hn,n=0,±1,±2,..., defined over the entire range of n. Then the linear convolution of discrete signals is ynof the form yn=k=−∞∞hkxn−k.

- Deprive the signal yn such that yn converges in the following cases.

  - Both hnand xnare causal.

  - The system has a finite impulse response such that hn=0,n<0∨n>M.

- Suppose the second case is hold, compute the convolution yn of xn=1↑,2,−3,2,1and hn=1↑,0,−1 using the “folding and shifting” method. Implement your answer with Scilab script.

- Suppose the second case is hold, compute the convolution yn of xn=1↑,2,−3,2,1and hn=1↑,0,−1using the matrix method. Implement your answer with Scilab script.

- Drawing xn, ynand energy of discrete signals in the same figure.

Exercise 7.

For a periodic signal xn of length N and impulse response hn of length M, the circular convolution between xnand hnis defined as yn=k=0M−1hkxn−k mod N. Explain why yncan be deprived that way.

- Compute the convolution yn of xn=1↑,2,−3,2,1and hn=1↑,0,−1,−1,1 using the “folding and shifting” method. Implement your answer with Scilab script.

- Compute the convolution yn of xn=1↑,2,−3,2,1and hn=1↑,0,−1,−1,1 using the matrix method. Implement your answer with Scilab script.

- Drawing xn, ynand energy of discrete signals in the same figure.

--------------- END ------------------